## Exercise 1: Sound Measurement

Goals

- Use the LabVIEW FPGA Toolkit and the NI SPI API to communicate with the microphone

- Simulate the SPI communication before compiling FPGA bitfile to verify functionality

- Plug in the acquisition code into the CompactRIO Waveform Library FPGA Template to handle scaling and data communication between the FPGA and LabVIEW Real-Time

### Part A – Application Description

In this exercise, use LabVIEW and the System on Module to acquire data from a microphone reference board over a SPI bus. Use the build in

### Part B – Code Implementation

#### Step 1

Reviewing the datasheet

The PmodMIC3 is a MEMS (Microelectromechanical System) microphone that makes us of an onboard ADC to sample analog audio signals. Once the ADC has digitized the analog signal, it transmits the data via a SPI bus to a receiving host device.

- Figure out Physical Connections

To use the Pmod with SOM, the pin configuration of the Pmod must be known before coding the interface.

  - Open the provided datasheet titled PmodMIC3_RM.pdf.

  - On page 2, the pinout for the Pmod is listed. Take note which pin corresponds to Signal Select/Chip Select (SS/CS), Serial Clock (SCK), and Master-In Slave-Out (MISO). For this application, we do not need to send information to the Pmod, so a MOSI line will not be used.

- Timing and SPI Configuration Settings

With the pin configuration known, the next important piece of information that is required is how the Pmod intends to transmits samples of data using these pins. In a typical SPI application, the toggling of the Signal Select (SS/CS) line initiates the transfer of information by informing all devices to transmit and listen on their corresponding MISO and MOSI lines. SPI is a shared clock protocol, meaning the Master device distributes one clock (SCK) that controls the transfer rate of all communication on the bus. Depending on device’s setup, a bit of data will be transmitted on the MISO and MOSI lines when there is either a rising or falling edge of SCK while the SS line is in its active state.

  - On page 2 of the data sheet, view the provided timing diagram and read the description of the Pmod’s communication procedure.

  - Important pieces of information to note:

    - Each bit is shifted out on each falling edge of SCK after CS is pulled low (active).

    - The ADC is 12-bit, but the transfer uses 16 bits (2 bytes). The first four bits are leading zeroes (unused) and the 12-bit sample fills the remaining 12 bits.

    - The data is sent with the MSB (most significant bit) first.

    - For fast transfers (higher SCK frequencies), SCK should remain at a default high state before SS is pulled low.

#### Step 2

- Add and setup the SOM hardware in LabVIEW

  - Open the provided LabVIEW project SOM Hands-On.lvproj

  - Add a SOM sbRIO-9651 to the project. Right-Click on Project: SOM Hands-On.lvproj (one line above My Computer) and select New > Targets and Devices.

  - In the window that appears, leave the default options selected and expand the Real-Time Single-Board RIO folder to discover the connected SOM sbRIO. When it appears select it and press OK. This will add a SOM target to your project view with a default name of RT Single-Board RIO Target.

  - Test to make sure you can connect to the target. Right-click on the SOM in the project and select Connect. A deployment window will appear and should mention that the deployment was successful. If it is not successful, alert your instructor.

  - Add the provided RT Main.vi to your target. Right-click on the SOM target and select Add > File, then choose RT Main.vi and press Add File.

  - Add an FPGA Target. Expand the SOM target, right-click on Chassis, and then select New > FPGA Target.

  - Add the SOM’s I/O. Under the FPGA Target, right-click on the sbRIO-9651 Socket and select Properties. In the new window that appears, within the General category, select DevKit from the sbRIO-9651 Socketed Component Level IP Declaration drop-down combo box. Press OK. In the project, you can now see all of the available I/O from the SOM daughter board populate under the Socket item.

  - Add a new 120 MHz derived clock. Under the FPGA Target, right-click the 40 MHz Onboard Clock and select New FPGA Derived Clock. In the window that appears, set the desired frequency to 120 MHz, and press OK. This clock will be used later in code to drive logic on the FPGA for the SPI communication.

  - Add the provided FPGA VIs to the project. Right-click on the FPGA Target and select Add > File, then select the FPGA Main_SPI API.vi and press Add File.

- Modify the SPI Example Code

  - Open the block diagram of FPGA Main_SPI API.vi.

  - View the layout of the code. There are three main parts to be aware of. First, the initialization code in the left-most section of the block diagram is responsible for designating which hardware I/O lines the SPI API will use to communicate. It also initializes FPGA memory items and FIFOs that the SPI API needs to operate. Next, the SPI Engine SCTL (single-cycle timed loop) is where the SPI Engine code will be placed. The SPI Engine is responsible for actually controlling, reading, and writing the I/O lines that were selected. Finally, the Command Queuing Loop is where commands will be prepared and transferred to the SPI Engine for execution.

  - Modify the DIO Lines and Enable Lines constants. In the initialization portion of the code, there are two cluster constants containing I/O references that can be used to select which pins the SOM should use for the SPI communication. In Step 1, the pinout of the PmodMIC3 was defined by the Pmod’s datasheet. This pinout will be used to determine the configuration of these I/O constants.

  - Turn on Context Help (Ctrl+H) and hover over each of the cluster constants. This will provide more information as to what each of the I/O constants is for. By default, the first I/O item of the DIO Lines cluster is SCLK followed by MISO, then MOSI, etc.

  - Press the drop-down arrow for each I/O item and select the resource that corresponds to the pin configuration from the PmodMIC3’s datasheet. For example, if the Pmod is plugged into the PMOD1 connector of the SOM’s daughter board, for SCLK the selected I/O item should be PMOD1_Pin4_out. This is because the PmodMIC3’s datasheet defines the 4th pin as SCLK. Repeat for MISO and CS 0 lines.

  - There are multiple CS lines available in the SPI API, but for this exercise only one CS line is required.

  - Wire up the SPI Engine as shown in the diagram using VIs from the SPI API palette.

  - Wire up the Command Queuing Loop as shown in the diagram.

- Simulate the VI in Desktop Execution Node

#### Step 3

- Compile the bitfile and test to see if the VI is operational

- Step 4